# Checklist BN-2 Islander

For use in flight simulator only! Do not use it in real world!

© 2004 by Marcel Kuhnt

### Before Bording:

Magnetos	OFF

Doors	OPEN

Bording	proceed

Doors	CLOSED and LOCKED

### Before Start:

Main/battery switch	ON

Fuel level	check

Fuel Selectors	Tank to Engine

Mixture pitch lever	MAX RICH

Prop pitch lever	MAX

Fuel pumps	ON

Magnetos	both engines LEFT

Beacon	ON

### Start

Starter switch	RIGHT

If right engine is running:

Starter switch	LEFT

If left engine is running:

Magnetos	both engines BOTH

Generators	both ON

Avionic Main Switches	ON

Fuel pressure	green arc

Fuel pumps	OFF

Oil temp and pressure	green arc

Nav lights, landing lights and passenger notices	ON as required

### Taxi

Parking brake	OFF

Brakes	check

### Before Take-Off

Fuel pumps	ON

Carb heat	ON, check carb temp., OFF

Flaps	T.O. position

Fuel level	checked

Prop & Magneto Checks:

RPM	2100

Prop pitch levers	three times each to FEATHER separately and back to MAX RPM

Magnetos	each engine BOTH-LEFT-BOTH-RIGHT, check RPM differences: max 175 from normal and max. 50 between LEFT and RIGHT

Throttle	Idle

Altitude indicator	adjust

Free/Slave switch (if fitted)	normally slave

if this is not fitted, heading indicator	adjust

Autopilot	OFF

### Take-Off

Throttle	Full

Rotate	61 KIAS

min. secure speed 50 ft. obstacle height	65 KIAS

After reaching 200 ft AGL, 70 KIAS and normal flight attitude:

Flaps	UP

Fuel pumps	OFF

### Climb

Throttle	for 26” manifold pressure

Prop pitch lever	for 2700 rpm

Mixture lever	lean above 2000-3000 ft MSL

Mind, that you have to increase the position of the throttle with higher altitude to hold the 26”!

### Cruise

Throttle	for 23” manifold pressure

Prop pitch lever	for 2300 rpm

Mixture lever	a bit more rich than max. EGT

Altitude indicator	adjust frequently

Heading indicator	slave or adjust frequently

Carburator heat	in wet / icing weather conditions use frequently; if manifold pressure decreases without climbing, switch it on for 30 seconds

### Descent

Rate of descent	500-700 ft/min

Throttle	for about 16” manifold pressure

Mixture	RICH

Carburator heat	ON

Fuel pumps	ON

Altitude indicator	adjusted

Heading indicator	slave or adjusted

### Approach

When the propellers fall under 2200 rpm:

Propeller	MAX RPM

Below 110 KIAS:

Flaps	T.O.

Below 80 KIAS:

Flaps	DOWN

Touch down speed	about 60-65 KIAS

### After Landing

Carburator heat	OFF

Fuel pumps	OFF

Flaps	UP

### After parking

Parking brake	ON

Throttle	IDLE

Mixture	SHUT OFF

Avionic master switch	OFF

Generators	OFF

Lights	OFF

Battery / main switch	OFF